C620

Brushless DC Motor Speed Controller

User Guide

使用说明

ユーザーガイド

V1.0

2017.09

Disclaimer

Thank you for purchasing the RoboMaster C620 Brushless DC Motor Speed Controller (hereinafter referred to as “product”). Read this disclaimer carefully before using this product. By using this product, you hereby agree to this disclaimer and signify that you have read it fully. Install and use this product in strict accordance with the User Guide. SZ DJI TECHNOLOGY CO., LTD.

and its affiliated companies assume no liability for damage(s) or injuries incurred directly or indirectly from using, installing or modifying this product improperly, including but not limited to using non-designated accessories.

DJITM is a trademark of SZ DJI TECHNOLOGY CO., LTD. (abbreviated as “DJI”) and its affiliated companies. Names of products, brands, etc., appearing in this document are trademarks or registered trademarks of their respective owner companies. This product and document are copyrighted by DJI with all rights reserved. No part of this product or document shall be reproduced in any form without the prior written consent or authorization of DJI.

The final interpretation rights of this disclaimer is owned by DJI.

Warning

- Make sure that there are no short circuits and all the cables are correctly connected when using the speed controller.

- The speed controller will heat when the output power is too high. Please handle carefully to avoid scalding.

- Ensure that all parts are in good condition. Replace them when necessary.

- The input signal mode (PWM signal or CAN bus communication) cannot be changed while the product is in use. Power off the product to set the signal mode and restart to apply a new mode.

- Be sure to use the product in strict accordance with the specifications (voltage, current, temperature, etc.) listed in this document. Failure to do so may reduce the product service life or even lead to permanent damage.

Introduction

Using a 32-bit motor driver chip and Field-Oriented Control (FOC), the RoboMaster C620 Brushless DC Motor Speed Controller enables precise control over motor torque. It is compatible with the M3508 P19 Brushless DC Gear Motor* to create a complete propulsion system. Users can configure and update speed controller firmware using RoboMaster Assistant.

* Please refer to the M3508 P19 Brushless DC Gear Motor User Guide for detailed information of this product.

Features

- Two Signal Modes

50-500Hz PWM (Pulse Width Modulation) signal CAN bus communication

- Maximum Continuous Current: 20 A

- Quick Setup to assign ID numbers to all speed controllers connected to the bus wire

- Feedback of motor temperature, rotor position and rotational speed via CAN bus communication

- No need for position sensor calibration when changing motors

Information about rotor position and rotational speed can only be obtained when using CAN bus communication. Please use the CAN bus cable reasonably while take its bandwidth into consideration.

In the Box

| C620 Speed Controller ×1 | PWM Cable×1 |
| --- | --- |
| CAN Cable ×1 | 7-Pin Cable ×1 |

Overview

C620 Brushless DC Motor Speed Controller

- 7-Pin Port

Connects to M3508 for data exchange.

- 3-Phase Power Cable

Connects to the 3-phase cable of the M3508. Ensure the cables are securely connected with the corresponding colors correctly matched.

- Termination Resistance Switch

Switch to connect/disconnect the 120Ω termination resistance, according to CAN bus wiring and relevant regulations.

- SET Button

Configures the speed controller. Refer to “Using the SET Button” for detailed information.

- Power Cable

Connects to a 24 V power supply via an XT30 power connector.

- Status LED

Indicates the speed controller’s operating status. Refer to “Status LED” for details.

- CAN Port

Receives the CAN controlling signal of the control panel. The bitrate of the CAN bus is 1 Mbps.

- PWM Port

Receives the PWM signal via a PWM cable. Connect to a DJI TAKYONTM Updater (or a compatible USB to serial converter) to your computer with the provided PWM cable to configure speed controller parameters or update its firmware via RoboMaster Assistant.

  - DO NOT plug in the CAN cable and the PWM cable simultaneously, as the motor may lose control. Make sure to power off the speed controller when switching the signal mode.

  - The DJI Takyon Updater should be purchased separately at the DJI online store. Refer to Takyon Updater User Guide for more information.

PWM Cable

A

B C

A: GND (Black) B: TX (Grey) C: PWM/RX (White)

CAN Cable

A

B

A: CAN_L (Black) B:CAN_H (Red)

Connecting the Speed Controller

Using CAN Cable

- Connect the 7-pin ports of the speed controller and M3508 with the 7-pin cable.

- Connect the 3-phase cable of the speed controller to that of the M3508. Ensure the cables are securely connected with the corresponding colors correctly matched.

- Insert one end of the CAN cable into the CAN port of the speed controller and the other end to any compatible equipment.

- Connect to a power supply.

7-Pin Cable

CAN Cable

3-Phase Cable

Power Supply Port

Using PWM Cable

- Connect the 7-pin ports of the speed controller and M3508 with the 7-pin cable.

- Connect the 3-phase cable of the speed controller to that of the M3508. Ensure the cables are securely connected with the corresponding colors correctly matched.

- Insert one end of the PWM cable into the PWM port on the speed controller and the other end to any compatible equipment.

- Connect to a power supply.

7-Pin Cable

Power Supply Port

3-Phase Cable

PWM Cable

Mounting the Speed Controller

25.80

18

Mounting Holes:

4 × 2.0

11.50

2.00

Refer to the dimensions below to mount the speed controller to a robot or mobile platform.

Top View

Side View

Unit: mm

The speed controller is outfitted with mounting holes of a diameter of 2.0. Use appropriately sized screws to mount properly and avoid damage.

Using RoboMaster Assistant

Configure the speed controller or update its firmware using a Takyon Updater (see below), or a compatible USB to serial converter via RoboMaster Assistant.

Speed Controller Takyon Updater  PC

- Download RoboMaster Assistant from the RoboMaster official website. www.robomaster.com/zh-CN/equipment/M3508

- Connect the speed controller to the Takyon Updater using the PWM cable, and then connect the Takyon Updater to a computer. Make sure the PWM cable is correctly connected; Black for GND (-), grey for TX (+) and white for PWM/RX ().

- Connect the speed controller to a power supply.

- Launch the RoboMaster Assistant and check if the speed controller is successfully connected to your computer.

- Click Settings to adjust parameters.

- Click Firmware Update and select the version you would like for the speed controller. RoboMaster Assistant will automatically download and update the firmware.

If RoboMaster Assistant does not recognize the speed controller, check:

if there is more than one FTDI device connected, such as another DJI Updater or program, an FTDI USB adapter, or development board (e.g., a BeagleBone, Raspberry, Arduino board). Unplug the FTDI devices. Restart RoboMaster Assistant and the speed controller, and try again.

Status LED

The chart below describes the blinking patterns of the speed controller’s operating status LED. In situations that indicate warning and abnormal working conditions, the LED will only indicate the abnormal one. And in situations that indicate multiple warning or abnormal working conditions, the LED will only indicate the one with the least amount of blinks. Please note that the speed controller will automatically cut off the output stream when it is in abnormal status.

| Normal | Description |
| --- | --- |
| Blinks green 1-8 times every second | The number of blinks indicate the current ID of the speed controller. |
| Quick ID Setting | Description |
| Solid orange | Quick ID setting starts |
| Motor Position Sensor Calibration | Description |
| Blinks green quickly | The motor position sensor is being calibrated. |

| PWM Calibration | Description |
| --- | --- |
| Solid Green | PWM signal is being calibrated |
| Blinks orange quickly | Calibration fails |
| Warning | Description |
| Blinks orange once every second | The temperature of motor is higher than or equal to 257° F (125° C)*. |
| Blinks orange twice every second | More than one speed controller share the same ID connected to the CAN bus. |
| Blinks orange three times every second | PWM output is not at the minimum |
| Abnormal | Description |
| Solid Red | Storage chip can not be accessed via 7-Pin cable or using incompatible motor (only check when powering on the speed controller). |
| Blinks red once every second | The voltage of the speed controller power supply is too high (only check when powering on the speed controller). |
| Blinks red twice every second | The 3-phase cable is not connected. |
| Blinks red three times every second | Motor position sensor data lost. Check that the 7-pin cable is connected correctly. |

| Blinks red four times every second | The temperature of the motor is abnormal or too high, higher than or equal to 356°F (180° F). |
| --- | --- |
| Blinks red quickly | Motor calibration fails |

* Temperature protection can be turned on/off via RoboMaster Assistant. The status LED will blink to warn and the output will be cut off when the motor temperature excesses 257° F if the temperature protection is activated. The status LED will not blink and the output will not be cut off when the motor temperature is between 257° F and 356° F if the temperature protection is inactivated.

Using the SET Button

- Separate ID Setting

Follow the steps below to assign a unique ID to each speed controller (supported ID range is 1-8):

  - Press the SET button once to activate separate ID assignment mode when the speed controller is working. The status LED will be off at that time.

  - Press the SET button again to assign an ID to each speed controller. The number of times you press the SET button (no more than eight times) is the ID number of the speed controller. The status LED will blink orange once for each successful press of the button.

  - If the SET button is idle for three seconds, the speed controller will automatically save the ID number entered. Turn off the speed controller and turn on again after assigning the ID.

Multiple speed controllers connected to the same CAN bus** cannot share the same ID number. Any speed controllers with the same ID number will indicate a warning and cut off the output stream.

- Quick ID Setting

Follow the steps below to quickly assign ID numbers to all speed controllers (eight maximum) connected to CAN bus**.

  - Ensure all speed controllers are working normally. Press the SET button on any speed controller speed controller once to activate separate ID assignment mode and then press and hold that SET button again until all the status LEDs turn solid green. All the speed controllers will assign ID numbers.

  - Turn each M3508 rotor manually (at least 180° in either direction) from first to last order of your choice. The ID number of each corresponding speed controller will be assigned, from one to eight (i.e., the first rotated motor means that its speed controller will have an ID of 1). Turn off the speed controllers and turn on again after assigning ID numbers.

If the M3508 rotor is not turned and fails to assign ID numbers to the speed controller, it will save the original ID number after powering on.

** Ensure to correctly connect or disconnect the terminal resistance based on CAN Bus wiring

standard, or the CAN BUS Communication may not work properly which leads to failure of those features.

- M3508 Calibration

For better performance and a smoother compatibility, please calibrate the M3508 after changing it or a speed controller. After the speed controller and M3508 are successfully connected and powered on, follow the steps below to calibrate the motor’s position sensor:

  - Press and hold the SET button and release when the status LED blinks green quickly.

  - The motor will begin calibration and complete when successful.

The motors will continue to rotate during calibration. DO NOT touch. It is recommend to calibrate the motors

without payload. If calibration fails several times, replace the motor.

CAN Communication Protocol

- Speed Controller Receiving Message Format The two identifiers (0x200 and 0x1FF) control the current output of each of the four speed controllers by ID numbers. The controllable current range is -16384 ~ 0 ~ 16384. The corresponding speed controller output torque current range is -20 ~ 0 ~ 20 A.

Identifier: 0x200	Frame format: DATA Frame type: Standard DLC: 8 Bytes

| Data Fields | Description | Speed Controller ID |
| --- | --- | --- |
| DATA[0] | Controls the current value in higher order byte (8 bits) | 1 |
| DATA[1] | Controls the current value in lower order byte (8 bits) |  |
| DATA[2] | Controls the current value in higher order byte (8 bits) | 2 |
| DATA[3] | Controls the current value in lower order byte (8 bits) |  |
| DATA[4] | Controls the current value in higher order byte (8 bits) | 3 |
| DATA[5] | Controls the current value in lower order byte (8 bits) |  |
| DATA[6] | Controls the current value in higher order byte (8 bits) | 4 |
| DATA[7] | Controls the current value in lower order byte (8 bits) |  |

Identifier: 0x1FF	Frame format: DATA Frame type: Standard DLC: 8 Bytes

| Data Fields | Description | Speed Controller ID |
| --- | --- | --- |
| DATA[0] | Controls the current value in higher order byte (8 bits) | 5 |
| DATA[1] | Controls the current value in lower order byte (8 bits) |  |
| DATA[2] | Controls the current value in higher order byte (8 bits) | 6 |
| DATA[3] | Controls the current value in lower order byte (8 bits) |  |
| DATA[4] | Controls the current value in higher order byte (8 bits) | 7 |
| DATA[5] | Controls the current value in lower order byte (8 bits) |  |
| DATA[6] | Controls the current value in higher order byte (8 bits) | 8 |
| DATA[7] | Controls the current value in lower order byte (8 bits) |  |

- Speed Controller Sending Message Format The format in which the speed controller sends feedback data to the CAN bus.

Identifier is determined by 0x200 + speed controller ID (e.g., if the speed controller ID is 1, then the identifier of that speed controller is 0×201)

Frame type: Standard Frame format: DATA DLC: 8 Bytes

| Data Fields | Description |
| --- | --- |
| DATA[0] | Controls the rotor mechanical angle in higher order byte (8 bits) |
| DATA[1] | Controls the rotor mechanical angle in lower order byte (8 bits) |
| DATA[2] | Controls the rotational speed in higher order byte (8 bits) |
| DATA[3] | Controls the rotational speed in lower order byte (8 bits) |
| DATA[4] | Actual torque current in higher order byte (8 bits) |
| DATA[5] | Actual torque current in lower order byte (8 bits) |
| DATA[6] | Motor temperature |
| DATA[7] | Null |

Sending frequency

1KHz by default (can be changed inside RoboMaster Assistant)

Rotor mechanical angle value range 0 ~ 8191 (corresponding mechanical angle range: 0 ~ 360°) Expressed rotor speed value unit rpm

Expressed motor temperature unit ℃

PWM Signal Description

- PWM Signal Control Modes

In one-way rotation mode, the diagram below shows the mapping between PWM pulse width and speed. At this time, the PWM signal can only rotate the motor in one direction.

Speed ( rpm )

1920

2000

1500

1000 1080

V max

0

Pulse Width ( us )

In two-way rotation mode, the diagram below describes the mapping between PWM pulse width and speed. In the first section, the speed controlled by pulse width decreases from the maximum speed in reverse to zero, and then accelerates from zero to max in a positive direction.

Speed ( rpm )

2000

1080	1480

1000	1500 1520

Pulse Width ( us )

1920

V max

0

V max

The default positive direction is counterclockwise to the rotation of the motor output shaft. Configure parameters related to PWM signal and calibrate the PWM signal inside RoboMaster Assistant.

- PWM Signal Range Calibration

When the input signal is set to be PWM (50 to 500Hz), its pulse width (1000 to 2000us) can be calibrated.

  - Connect the speed controller and a PWM equipment via the provided PWM cable.

  - Adjust the pulse width to the maximum and power on the speed controller and the motor. The PWM signal range calibration starts with the status LED being solid green.

  - Adjust the pulse width to the minimum within 3 seconds. The speed controller will work normally if calibration successes with the status LED being green; If calibration fails, the status LED will blink orange quickly.

Performance Diagram

The following data displays the performance of the M3508 P19 Brushless DC Gear Motor and was generated using the C620:

n(rpm) η(%)

I(A) P(W)

I

P

η

n

450  80

400  70

350  60

300  50

250  40

200

150  30

100  20

50  10

0  0

1  2  3  4  5

16

14

12

10

8

6

4

2

0

6

T(N·m)

250

200

150

100

50

0

η – Electrical Efficiency, T – Thrust, I – Current, P – Output Power, n – Rotational Speed

The data above was measured in a laboratory setting with an input voltage of 24 V, at a temperature of 25°C and under well-ventilated conditions. These figures should be used for reference only. Make sure to use the speed controller properly, depending on the actual temperature and cooling conditions.

Specifications

| Rated Voltage | 24 V |
| --- | --- |
| Max Allowable Current* (continuous) | 20 A |
| Max PWM Input Signal Frequency | 50 to 500 Hz |
| CAN Bus Bitrate | 1 Mbps |
| Weight | 35 g |
| Dimensions | 49.4×25.8×11.5 mm (LWH) |
| Operating Temperature Range | 32 to 122° F (0 to 50° C) |

* Tested at a temperature of 25° C in well-ventilated laboratory conditions.